# AI Framework, Model & Explanation — ACN1408 (for Complete Specification)

# AI Framework & Model — Disclosure Text for the Complete Specification (ACN1408)

*Drop-in technical-disclosure section for the Form-2 Complete Specification. Patent-style,

enablement-level. Reference numerals match reference_numerals.md and Figures 1–5. Where

the as-built embodiment differs from the provisional, see provisional_reconciliation.md

(the patent agent should reconcile the claim language accordingly).*

## 1. System overview (FIG. 1)

The AI-based alcohol detection and vehicle ignition prevention system (100) comprises a

wearable smartwatch device (110) and a vehicle control module (150) coupled by a secure

Bluetooth Low Energy link (140). The smartwatch (110) carries a photoplethysmography

sensor (112), an electrodermal-activity channel (114), and a skin/ambient temperature

sensor (116). An on-device inference engine (118) executes an AI/ML module (120) that

estimates blood alcohol concentration (BAC) from the sensed physiological signals.

The estimated BAC, together with a confidence value and status flags, is transmitted as a

periodic status packet over the secure link (140) to the vehicle control module (150),

which comprises a BLE receiver (152), fail-safe decision logic (154), and a relay/MOSFET

ignition switch (156) coupled to the vehicle ignition / OBD-II / ECU interface (160).

Status indicators (170) and a manual override button (180) are provided. The decision

logic (154) is fail-safe: ignition is blocked by default and is enabled only while all

permissive conditions hold.

## 2. AI framework / signal-processing pipeline (FIG. 3)

In operation, raw signals from the sensors (112, 114, 116) are preprocessed and segmented

into fixed-length temporal windows of ten time steps (122), forming a feature tensor (124)

of shape [10 time steps × 6 features]. The six features comprise a PPG value, a PPG signal

quality measure, an electrodermal-activity value, a skin temperature, an ambient

temperature, and a humidity value. The feature tensor (124) is supplied to a trained

neural network (126, 128, 130) that produces a raw BAC estimate.

The raw BAC estimate is then adjusted by a climate-adaptive calibration stage (132). The

calibration (132) is a deterministic post-inference correction applied to the model

output using region-specific temperature and humidity coefficients; it is not a layer

of the trained neural network. The resulting calibrated BAC is compared by the decision

logic (154) against a legal threshold of 0.08 g/dL; the allow/block determination, with a

confidence value, is transmitted over the secure link (140) to control the ignition (160).

## 3. Neural-network model architecture (FIG. 4)

The trained model receives the feature tensor (124) of shape [B, 10, 6] and processes it

through a bidirectional long short-term memory (BiLSTM) stage (126) of 64 units, producing

a temporally-encoded representation of shape [B, 10, 128], followed by a dropout stage

(rate 0.3).

A temporal attention mechanism (128) operates on the encoded sequence: a dense projection

with a hyperbolic-tangent activation computes per-time-step scores, a softmax produces

attention weights over the ten time steps, and a weighted sum across time produces a

128-dimensional attention context vector. The context vector is passed through a dense

regression head (130) — a 32-unit rectified-linear layer, a dropout stage (rate 0.3), a

16-unit rectified-linear layer, and a final single-unit linear layer — that outputs the

BAC estimate in g/dL.

Safety-prioritized training. The model is trained with an asymmetric loss function in

which false-negative errors (predicting a sober reading when the true BAC is above the

threshold) are penalized 30× more heavily than false-positive errors. This biases the

system toward the safety-critical suppression of missed-intoxication events.

On-device deployment. The trained model is quantized and deployed on the smartwatch as

a TensorFlow Lite model executed by the on-device inference engine (118). Because the

network contains LSTM operations, the converted model retains the requisite TensorFlow

Lite operator set (SELECT_TF_OPS) for on-device execution.

## 4. System operation and verification sequence (FIG. 2)

The smartwatch (110) continuously supplies sensor data to the AI/BAC module (120), which

estimates BAC and compares it to the 0.08 g/dL threshold and transmits a BAC status packet

over the secure link (140) at a regular interval. The vehicle module (150) blocks ignition

by default; upon an override attempt it requests verification, prompting a fresh

sensor-data recheck, and otherwise maintains the ignition block.

## 5. Fail-safe vehicle-control state machine (FIG. 5)

The decision logic (154) implements a fail-safe state machine. The initial/default state

upon power-on is IGNITION_BLOCKED. Ignition is enabled (IGNITION_ALLOWED) only while the

link is active, a recent BAC update has been received, the estimated BAC is below 0.08

g/dL, and the watch is worn. Loss of communication beyond a timeout transitions to

CONNECTION_LOST and re-blocks; removal of the watch or stale data re-blocks. A manual

override (180), engaged by a sustained button hold, transitions to a time-limited, logged

OVERRIDE_ACTIVE state that automatically reverts to IGNITION_BLOCKED.

## Notes for the patent agent (accuracy)

The electrodermal-activity channel (114) is derived/estimated from heart-rate

variability in the present embodiment (most consumer smartwatches lack a dedicated EDA

sensor); it is not represented as a directly-sensed EDA measurement.

BAC is inferred from physiological proxies by the trained model; the present

embodiment does not use a transdermal alcohol sensor.

The model is trained on physiologically-modeled synthetic data in the present

embodiment.

The 0.08 g/dL threshold is a fixed legal threshold in the present embodiment.

The secure link (140) uses AES-256 per spec; its implementation status is addressed in

provisional_reconciliation.md.