# Provisional vs As-Built Reconciliation — ACN1408

# Provisional ↔ As-Built Reconciliation — ACN1408

For the patent agent. This note lists points where the as-built embodiment (the

AlcoWatch code: ml_model/, wear_os_app/, arduino/, and CLAUDE.md) differs from, or

is narrower than, statements in the provisional specification (ACN1408 Prov._Patent_130625).

A Complete Specification that mirrors the code may otherwise internally contradict its own

provisional or claim subject matter the embodiment does not support — an enablement /

best-mode risk an examiner or opponent can exploit. For each item, decide whether to (a)

keep the code-faithful framing used in disclosure_ai_section.md, or (b) amend the

provisional/claim language. This package does not resolve these — it surfaces them.

| # | Provisional says | As-built embodiment | Recommended framing |
| --- | --- | --- | --- |
| 1 | A smartwatch with an EDA sensor "for quantifying sympathetic nervous system responses" | No dedicated EDA sensor; EDA is estimated/derived from heart-rate variability (most Wear OS devices lack EDA hardware) | Describe EDA (114) as estimated/derived; if claims require a literal EDA sensor, narrow or mark as an optional alternative embodiment |
| 2 | AI infers "transdermal alcohol concentrations" which are converted to BAC | No transdermal alcohol sensor; BAC is regressed from physiological proxies (PPG/EDA/temperature) by the model | Describe BAC as inferred from physiological proxies; avoid asserting transdermal sensing as implemented |
| 3 | Models "trained on large-scale physiological datasets" | Trained on physiologically-modeled synthetic data in the present embodiment | State the synthetic-data basis honestly (best mode); large-scale real datasets may be claimed as a further embodiment |
| 4 | AI that "dynamically adjusts / personalizes BAC thresholds to individual baselines" | Fixed legal threshold of 0.08 g/dL; region (not individual) calibration coefficients | Either disclose the fixed-threshold embodiment, or keep dynamic/personalized thresholds as a claimed alternative not asserted as built |
| 5 | Provisional Figure 1 (and AI module) depicts "Security and Biometric Authentication" | No biometric-authentication function in the model/code | Omit biometric authentication from the redrawn Figure 1 (done) unless it is added to the embodiment; do not claim it as implemented |
| 6 | Communication uses AES-256 encryption | AES-256 is specified but the current implementation uses basic BLE pairing | Label the link (140) "secure ... AES-256 per spec" (done); disclose AES-256 as the specified/target security, not as the as-shipped implementation |

## Items confirmed consistent (no action)

Model architecture (FIG. 4) matches the code exactly: BiLSTM(64) → [B,10,128],

Dropout(0.3), temporal attention (Dense-tanh → softmax → weighted sum), Dense 32/16/1.

Asymmetric loss penalizing false negatives 30× (code: * 30.0).

Climate-adaptive calibration is a real post-inference correction (code:

ClimateAdaptiveModel.calibrate_prediction; on-device applyClimateCalibration).

Fail-safe default-BLOCK ignition logic (FIG. 5) matches the firmware state machine.

0.08 g/dL legal threshold matches the hardcoded LEGAL_BAC_LIMIT.